TRAPNET MASTER CURRICULUM & AI ENGINEERING PLAYBOOK
TRAPNET Core Concepts & Learning Mastery

Comprehensive guide covering Embedded Microcontrollers, Edge AI, Multi-Modal Sensor Fusion, Power Systems, and AI Collaboration Strategy.

# 1. Embedded Microcontrollers & Real-Time Firmware (ESP32)

To understand what you were doing in TRAPNET, master these 4 low-level embedded hardware concepts:

FreeRTOS Dual-Core Multitasking: The ESP32 WROOM-32 houses two 32-bit Tensilica LX6 CPU cores operating at 240MHz. In TRAPNET, Core 0 handles wireless communication & background sensor polling (mmWave radar & GSM), while Core 1 handles real-time interrupt processing & relay triggers.

GPIO Dynamics & Boot Restrictions: Not all GPIO pins are equal. GPIO34, 35, 36, and 39 are Input-Only pins requiring external 10kΩ pull-down resistors. Boot pins (GPIO0, 2, 12, 15) determine flashing vs normal operation; GPIO12 must stay LOW at boot to avoid flash power voltage errors.

Hardware Serial/UART Buses: TRAPNET uses two dedicated UART buses: UART2 (GPIO16/17 @ 256000 baud) for high-speed mmWave radar binary frames, and SoftSerial UART1 (GPIO26/27 @ 9600 baud) for AT commands to the SIM800L GSM module.

Hardware Interrupts & Debouncing: Perimeter sensors (IR door/window break & SW-420 vibration) trigger non-blocking hardware interrupts on pin state change rather than wasteful continuous CPU polling.

# 2. Radar Physics & Multi-Modal Sensor Fusion

24GHz FMCW mmWave Radar Principles: The HLK-LD2410C transmits Frequency Modulated Continuous Wave signals at 24GHz. It measures target motion, distance, and energy gates. It can detect static micro-movements like human breathing even when an intruder stays completely still.

Spatial Vision-Radar Fusion Math: Single-modality sensors fail easily (cameras fail in fog/darkness; radar fails on static pets). TRAPNET cross-verifies camera optical range against radar range using the pinhole formula: D_vision = (f * H) / h. If |D_vision - D_radar| <= 25cm, the threat is confirmed.

False Positive Suppression: Algorithmic energy gating eliminates false alarms caused by insects, moving foliage, wind curtains, or small pets.

# 3. Edge AI & Mobile Computer Vision (YOLOv8)

YOLOv8 Single-Stage Architecture: YOLOv8 uses an anchor-free split head design that predicts bounding boxes and object classes simultaneously in a single forward pass.

INT8 Model Quantization: Converting 32-bit floating-point weights to 8-bit integers (INT8 Post-Training Quantization) reduces model memory size by 75% and accelerates execution on mobile NPU hardware by 4x to 5x.

Android Mobile NPU Acceleration: Executing TFLite models via Android NNAPI (Neural Networks API) leverages Qualcomm Hexagon DSP or Mobile GPU hardware, dropping inference latency from 150ms (Raspberry Pi) down to sub-15ms.

# 4. How to Use AI (Antigravity/Gemini) Wisely as an Engineer

Provide Complete Technical Context: Never ask vague questions like 'How to connect radar?'. Always specify exact chip revisions, pin numbers, baud rates, and library versions.

Use Planning Mode First: Ask for architecture diagrams, data flow schemas, and edge-case failure modes before asking for implementation code.

Paste Raw Stack Traces: When code fails to compile or crashes, paste un-truncated compiler logs so the AI pinpoints the exact line number and variable error.

Step-by-Step Empirical Verification: Test generated code modularly on real hardware before building full system integration.